- MM Format (Used for D-Series, A-Series and Acecat I, II, III)

- Format: RS-232-C

Baud Rate: 9600					Number of Data Bits: eight

Parity bit: odd						Number of stop bits: one

Data Format:

---------------------------------------------------------------------------------------

Transmit	|	MSB												LSB

Sequence	|	7		6		5		4		3		2		1		0

---------------------------------------------------------------------------------------

Byte 1		|	1		PR		T		Sx		Sy		B2		B1		B0

Byte 2		|	0		X6		X5		X4		X3		X2		X1		X0

Byte 3		|	0		X13	X12	X11	X10	X9		X8		X7

Byte 4		|	0		Y6		Y5		Y4		Y3		Y2		Y1		Y0

Byte 5		|	0		Y13	Y12	Y11	Y10	Y9		Y8		Y7

---------------------------------------------------------------------------------------

PR			: 1 = Out proximity, 0 = In proximity

B0 ~ B3	: Button state

T			: Tablet ID

- Power ON default: MM Format, Point Mode 3, 500 LPI

=============================================================

=============================================================

- ACECAD Format (for Acecat Flair)

- Format: RS-232-C

Baud Rate: 9600

Number of Data Bits: eight

Parity bit: odd

Number of stop bits: one

Data Format:

---------------------------------------------------------------------------------------

Transmit	|	MSB												LSB

Sequence	|	7		6		5		4		3		2		1		0

---------------------------------------------------------------------------------------

Byte 1		|	1		PR		T		Sx		Sy		B2		B1		B0

Byte 2		|	0		X6		X5		X4		X3		X2		X1		X0

Byte 3		|	0		X13	X12	X11	X10	X9		X8		X7

Byte 4		|	0		Y6		Y5		Y4		Y3		Y2		Y1		Y0

Byte 5		|	0		Y13	Y12	Y11	Y10	Y9		Y8		Y7

Byte 6		|	0		R8		R7		R6		R5		R4		R3		R2

Byte 7		|	0		0		0		R0		Cr1		Cr0		B3		R1

----------------------------------------------------------------------------------------

PR			: 1 = Out proximity, 0 = In proximity

B0 ~ B3	: Button state

T			: Tablet ID

R0 ~ R8	: Pen tip pressure value

Cr1, Cr0	: 0,0 = 4 Button puck

: 0,1 = 3 Button stylus pen (default)

: 1,0 = 16 Button puck

: 1,1 = Reserved

- Power ON default: ACECAD Format, Increment Mode 3, 1000 LPI

=============================================================

=============================================================

ASCII – Absolute Coordinate Mode (Support by A-Series and Acecat I,II,III)

-----------------------------------------------------------------------

Resolution < 508 LPI			XXXX,YYYY,F CR LF

Resolution > 508 LPI			XXXXX,YYYYY,F CR LF

-----------------------------------------------------------------------

ASCII – Delta Coordinate Mode (Support by A-Series and Acecat I,II,III)

--------------------------------------------------------------------------

Resolution < 508 LPI			SXXXX,SYYYY,F CR LF

Resolution > 508 LPI			SXXXXX,SYYYYY,F CR LF

--------------------------------------------------------------------------

=============================================================

=============================================================

Command:

Reset								<Null> (00h)

*Note: The Acecat Flair also support the ‘z0’ command to reset the tablet. The

‘z0’ command not only reset the Flair, but also returns the ACECAD Tablet

Identify Information String. For the descriptions to the content of the datas,

Please refer to the following section “ACECAD Tablet Identify Information

String”.

Send Configuration				a

Coordinate System:

Absolute						F

Relative						E

Report Modes:

Point Mode					B

Remote Request Mode:

mode command			D

trigger command			P

Stream Mode					@

Switch Stream Mode			A

Increment Mode:

Command						I

Increment value				<SP> to <DEL>

Report Rate:

120 rps							Q

85 rps							R

10 rps							S

2 rps							T

Resolution Predefined:

1 lpi							l

2 lpi							n

4 lpi							p

100 lpi							d

200 lpi							e

10 lpmm(254 lpi)				f

400 lpi							g

500 lpi							h

20 lpmm(508 lpi)				i

1000 lpi						j

40 lpmm(1016 lpi)				q

2000 lpi						s

2540 lpi						v

Stop Transmission (XOFF)		<CTRL>S

Resume Transmission (XON)		<CTRL>Q

ASCII BCD report format			za

Binary report format				zb

=============================================================

=============================================================

- ACECAD Tablet Identify Information String (for Acecat III and Acecat Flair)

- Command: ‘z~’

Returned data: (Total 17 bytes)

- ACECAD ID header string------------------------------------6 Byte

Content: 41h,43h,45h,43h,41h,44h			; “ACECAD”

- Model-------------------------------------------------------------2 Byte

Content: 54h,30h							; “T0”, meaning Acecat III

46h,30h							; “F0”, meaning Flair

- Version------------------------------------------------------------1 Byte

Content: bit 7  6  5  4  3  2  1  0

M			N			; M.N represent the version

- Interface-----------------------------------------------------------1 Byte

Reserved.

- Sample Rate Count Frequency---------------------------------1 Byte

Reserved.

- Resolution Calculation Coefficient----------------------------2 Byte

Reserved.

- Cordless or Cableless--------------------------------------------1 Byte

Reserved.

- Transducer Type--------------------------------------------------1 Byte

Reserved.

- Pressure Support--------------------------------------------------1 Byte

Reserved.

- Pressure Threshold----------------------------------------------1 Byte

Reserved.

- Data Format------------------------------------------------------1 Byte

Definition:	00h: MM binary ---------------Power on default

01h: MM ASCII

02h: UIOF binary

03h: UIOF ASCII

- Sample Rate------------------------------------------------------2 Byte

Reserved.

- Resolution of X--------------------------------------------------2 Byte

Reserved.

- Resolution of Y--------------------------------------------------2 Byte

Reserved.

- INC Value--------------------------------------------------------1 Byte

Definition: 	Represent the increment threshold value. Data

Packet will be sent if the movement is greater

than the threshold.

Range = 00h ~ 0ffh

- Work Mode-------------------------------------------------------1 Byte

Definition:	Bit 7: 1 if origin at upper left

Bit 6: 1 if ACECAT is emulating 12”x12”

Bit 5: 1 if Remote mode is selected

Bit 4: 1 if Stream mode is selected

Bit 3: 1 if Axis Update mode is selected

Bit 2: 1 if Track mode is selected ----------------Power on default

Bit 1: 1 if Point mode is selected

Bit 0: 1 if Increment mode is selected

- END Signal-------------------------------------------------------2 Byte

Represent the end of ID string, always send two bytes: 55h,55h

Note:

To check the ACECAD ID string, follow the following steps:

- Send the ‘z~’ to the tablet.

- Receive the data and save to buffer.

- Counting the number of data received. The correct number is 17.

- Check whether the last two bytes (END Signal) are 5555h.

- Check the first item (ACECAD ID header string).

Normally, the ACECAD tablet detection is finished here. The other items are used for

configuration checking. For example, if you set the tablet to the other working mode, you

may need to verify whether it is changed or not.